Assignment 1

| Course Code: | IT2B (Network) |
| --- | --- |
| Course Name: | Network Administration |
| Assignment: | CIDR Assignment |

| Textbook: | Lammle, Todd. (2021). CompTIA Network+ Study Guide (5e). Sybex. |
| --- | --- |

Materials and Resources

Assignment Description

In this assignment, you will solve CIDR-related problems.

Assignment Steps

Activity Steps

- Binary-Decimal Number Conversion (12 marks) - Convert the following decimal numbers to binary. 88 = 01011000 (Written remainder from bottom to top and add 0 ahead to complete 8 bit.) Formula for conversion: Dividend; Remainder. 88/2 = 44; 0. 44/2 = 22; 0. 22/2 = 11; 0. 11/2 = 5; 1. 5/2 = 2; 1. 2/2 = 1; 0. ½ = 0; 1. 241 = 11110001 Dividend; Remainder. 241/2 = 120; 1. 120/2 = 60; 0. 60/2 = 30; 0. 30/2 =15; 0. 15/2 = 7; 1. 7/2 = 3; 1. 3/2 = 1; 1. ½ = 0; 1. 99 =01100011 Dividend; Remainder. 99/2 = 49; 1. 49/2 = 24; 1. 24/2 = 12; 0. 12/2 = 6; 0. 6/2 = 3; 0. 3/2 = 1; 1. ½ = 0; 1. 42 =00101010 Dividend; Remainder. 42/2 = 21; 0. 21/2 = 10; 1. 10/2 = 5; 0. 5/2 = 2; 1. 2/2 = 1; 0. ½ = 0; 1. 169 =10101001 Dividend; Remainder. 169/2 = 84; 1. 84/2 = 42; 0. 42/2 = 21; 0. 21/2 = 10; 1. 10/2 = 5; 0. 5/2 = 2; 1. 2/2 = 1; 0. ½ = 0; 1. 311 =100110111 Dividend; Remainder. 311/2 = 155; 1. 155/2 = 77; 1. 77/2 = 38; 1. 38/2 =19; 0. 19/2 = 9; 1. 9/2 = 4; 1. 4/2 = 2; 0. 2/2 = 1; 0. ½ = 0; 1. - Convert the following binary numbers to decimal. 2^7; 2^6; 2^5; 2^4; 2^3; 2^2; 2^1; 2^0. 128; 64; 32; 16; 8; 4; 2; 1. 01011100 = ( 64+16+8+4 ) 92 00111100 = (32+16+8+4) 60 11110000 = (128+64+32+16) 240 10101011 = (128+32+8+2+1) 171 11001011 = (128+64+8+2+1)203 01111101 = (64+32+16+8+4+1)125

- Subnetting Class A, B, and C Networks (30 marks) Subnet the following networks (using 2n = number of subnets) – figure out the subnet mask required, the first 3 usable ranges, and the number of hosts per subnet. 3.0.0.0/8 network (with 6 subnets) (5 marks) Solution(1) Given Network = 3.0.0.0/8 belongs to Class A. Subnet Mask by default for Class A = 255.0.0.0 (11111111/00000000/00000000/00000000) 8 bits are for Network and remaining 24 bits are for Host. Network ID = 3.0.0.0/8 Calculate New Subnet Mask with 6 subnets: Formula 2^n >= 6 we need 3 bits (since 2^3=8, which is greater than or equal to 6). Therefore, Subnet Mask changed from : 255.0.0.0 (11111111/00000000/00000000/00000000) to 255.224.0.0 (11111111/11100000/00000000/00000000) Calculate No of Host : 2^n-2 2^21-2= 2097150 First 3 usable IP ranges: First Subnet: Network ID = 3.0.0.0/11 Host ID Range = 3.0.0.1 to 3.0.31.254 Broadcast Address = 3.0.31.255 Second Subnet: Network ID = 3.0.32.0/11 Host ID Range = 3.0.32.1 to 3.0.63.254 Broadcast Address = 3.0.63.255 Third Subnet: Network ID = 3.0.64.0/11 Host ID Range = 3.0.64.1 to 3.0.95.254 Broadcast Address = 3.0.95.255 192.168.7.0/24 network (with 11 subnets) (5 marks) Solution(2) Given Network = 192.168.7.0/24 belongs to Class C. Subnet Mask by default for Class C = 255.255.255.0 (11111111/11111111/11111111/00000000) 24 bits are for Network and remaining 8 bits are for Host. Network ID = 192.168.7.0/24 Calculate New Subnet Mask with 11 subnets: Formula 2^n >= 11 we need 4 bits (since 2^4=16, which is greater than or equal to 11). Therefore, Subnet Mask changed from : 255.255.255.0 (11111111/11111111/11111111/00000000) to 255.255.255.240 (11111111/11111111/11111111/11110000) Calculate No of Host : 2^n-2 2^4-2= 14 First 3 usable IP ranges: First Subnet: Network ID = 192.168.7.0/28 Host ID Range = 192.168.7.1 to 192.168.7.14 Broadcast Address = 192.168.7.15 Second Subnet: Network ID = 192.168.7.16/28 Host ID Range = 192.168.7.17 to 192.168.7.30 Broadcast Address = 192.168.7.31 Third Subnet: Network ID = 192.168.7.32/28 Host ID Range = 192.168.7.33 to 192.168.7.46 Broadcast Address = 192.168.7.47 131.19.0.0/16 network (with 2 subnets) (5 marks) Solution(3) Given Network = 131.19.0.0/16 belongs to Class B. Subnet Mask by default for Class B = 255.255.0.0 (11111111/11111111/00000000/00000000) 16 bits are for Network and remaining 16 bits are for Host. Network ID = 131.19.0.0/16 Calculate New Subnet Mask with 2 subnets: Formula 2^n >= 2 we need 1 bit (since 2^1=2, which is greater than or equal to 2). Therefore, Subnet Mask changed from : 255.255.0.0 (11111111/11111111/00000000/00000000) to 255.255.128.0 (11111111/11111111/10000000/00000000) Calculate No of Host : 2^n-2 2^15-2= 32766 First 3 usable IP ranges: First Subnet: Network ID = 131.19.0.0/17 Host ID Range = 131.19.0.1 to 131.19.127.254 Broadcast Address = 131.19.127.255 Second Subnet: Network ID = 131.19.128.0/17 Host ID Range = 131.19.128.1 to 131.19.255.254 Broadcast Address = 131.19.255.255 111.0.0.0/8 network (with 15 subnets) (5 marks) Solution(4) Given Network = 111.0.0.0/8 belongs to Class A. Subnet Mask by default for Class A = 255.0.0.0 (11111111/00000000/00000000/00000000) 8 bits are for Network and remaining 24 bits are for Host. Network ID = 111.0.0.0/8 Calculate New Subnet Mask with 15 subnets: Formula 2^n >= 2 we need 4 bits (since 2^4=16, which is greater than or equal to 15). Therefore, Subnet Mask changed from : 255.0.0.0 (11111111/00000000/00000000/00000000) to 255.240.0.0 (11111111/11110000/00000000/00000000) Calculate No of Host : 2^n-2 2^20-2= 1048574 First 3 usable IP ranges: First Subnet: Network ID = 111.0.0.0/12 Host ID Range = 111.0.0.1 to 111.15.255.254 Broadcast Address = 111.15.255.255 Second Subnet: Network ID = 111.16.0.0/12 Host ID Range = 111.16.0.1 to 111.31.255.254 Broadcast Address = 111.31.255.255 Third Subnet: Network ID = 111.32.0.0/12 Host ID Range = 111.32.0.1 to 111.47.255.254 Broadcast Address = 111.47.255.255 177.2.0.0/16 network (with 21 subnets) (5 marks) Solution(5) Given Network = 177.2.0.0/16 belongs to Class B. Subnet Mask by default for Class B = 255.255.0.0 (11111111/11111111/00000000/00000000) 16 bits are for Network and remaining 16 bits are for Host. Network ID = 172.2.0.0/16 Calculate New Subnet Mask with 21 subnets: Formula 2^n >= 2 we need 5 bits (since 2^5=32, which is greater than or equal to 21). Therefore, Subnet Mask changed from : 255.255.0.0 (11111111/11111111/00000000/00000000) to 255.255.248.0 (11111111/11111111/11111000/00000000) Calculate No of Host : 2^n-2 2^11-2= 2046 First 3 usable IP ranges: First Subnet: Network ID = 177.2.0.0/21 Host ID Range = 172.2.0.1 to 172.2.7.254 Broadcast Address = 172.2.7.255 Second Subnet: Network ID = 177.2.8.0/21 Host ID Range = 172.2.8.1 to 172.2.15.254 Broadcast Address = 172.2.15.255 Third Subnet: Network ID = 177.2.16.0/21 Host ID Range = 172.2.16.1 to 172.2.23.254 Broadcast Address = 172.2.23.255 220.13.42.0/24 network (with 3 subnets) (5 marks) Solution(6) Given Network = 220.13.42.0/24 belongs to Class C. Subnet Mask by default for Class B = 255.255.255.0 (11111111/11111111/11111111/00000000) 24 bits are for Network and remaining 8 bits are for Host. Network ID = 220.13.42.0/24 Calculate New Subnet Mask with 3 subnets: Formula 2^n >= 2 we need 2 bits (since 2^2=4, which is greater than or equal to 3). Therefore, Subnet Mask changed from : 255.255.255.0 (11111111/11111111/11111111/00000000) to 255.255.255.172 (11111111/11111111/11111111/11000000) Calculate No of Host : 2^n-2 2^6-2= 62 First 3 usable IP ranges: First Subnet: Network ID = 220.13.42.0/26 Host ID Range = 220.13.42.1 to 220.13.42.62 Broadcast Address = 220.13.42.63 Second Subnet: Network ID = 220.13.42.64/26 Host ID Range = 220.13.42.65 to 220.13.42.126 Broadcast Address = 220.13.42.127 Third Subnet: Network ID = 220.13.42.128/26 Host ID Range = 220.13.42.129 to 220.13.42.190 Broadcast Address = 220.13.42.191

- Difficult Subnet Examples (8 marks) Subnet the following network (using 2n = number of subnets) – figure out the subnet mask required, the first and last 3 usable ranges, and the number of hosts per subnet. 101.0.0.0/8 network (with 1300 subnets) Solution Given Network = 101.0.0.0/8 belongs to Class A. Subnet Mask by default for Class A = 255.0.0.0 (11111111/00000000/00000000/00000000) 8 bits are for Network and remaining 24 bits are for Host. Network ID = 101.0.0.0/8 Calculate New Subnet Mask with 1300 subnets: Formula 2^n >= 2 we need 11 bits (since 2^11=2048, which is greater than or equal to 1300). Therefore, Subnet Mask changed from : 255.0.0.0 (11111111/00000000/00000000/00000000)to 255.255.224.0 (11111111/11111111/11100000/00000000) Calculate No of Host : 2^n-2 2^13-2= 8190 First 3 usable IP ranges: First Subnet: Network ID = 101.0.0.0/19 Host ID Range = 101.0.0.1 to 101.0.31.254 Broadcast Address = 101.0.31.255 Second Subnet: Network ID = 101.0.32.0/19 Host ID Range = 101.0.32.1 to 101.0.63.254 Broadcast Address = 101.0.63.255 Third Subnet: Network ID = 101.0.64.0/19 Host ID Range = 101.0.64.1 to 101.0.95.254 Broadcast Address = 101.0.95.255 Last 3 usable subnets: Last 3rd Subnet: Network ID = 101.255.96.0/19 Host ID Range = 101.255.96.1 – 101.255.127.254 Broadcast Address = 101.255.159.255 Second last Subnet: Network ID = 101.255.128.0/19 Host ID Range = 101.255.128.1 – 101.255.159.254 Broadcast Address = 101.255.159.255 last Subnet: Network ID = 101.255.160.0/19 Host ID Range = 101.255.160.1 – 101.255.191.254 Broadcast Address = 101.255.191.255

- Identifying the Broadcast ID of a Subnet. (7 marks) - Identify the broadcast ID for the 2nd usable subnet in all exercises in Question 2. 3.0.0.0/8 network (with 6 subnets) Broadcast Address 2nd usable subnet = 3.0.63.255 192.168.7.0/24 network (with 11 subnets) Broadcast Address2nd usable subnet = 192.168.7.31 131.19.0.0/16 network (with 2 subnets) Broadcast Address2nd usable subnet = 131.19.255.255 111.0.0.0/8 network (with 15 subnets) Broadcast Address 2nd usable subnet = 111.31.255.255 177.2.0.0/16 network (with 21 subnets) Broadcast Address 2nd usable subnet = 172.2.15.255 220.13.42.0/24 network (with 3 subnets) Broadcast Address 2nd usable subnet = 220.13.42.127 - Identify the broadcast ID for the 2nd usable subnet in Question 3. Broadcast Address = 101.0.63.255

- Identify whether the following hosts are on the same subnet (or not). (6 marks) 192.168.7.62/28 and 192.168.7.70/28 Solution : Step 1: /28 Subnet Mask A /28 subnet mask is 255.255.255.240. The first 28 bits represent the network portion and 4 bits represent the host portion. Step 2: Find the Subnet Range for 192.168.7.62/28 IP Address: 192.168.7.62 Subnet Mask: 255.255.255.240 0-15 = Network 192.168.7.0/28 16-31 = Network 192.168.7.16/28 32-47 = Network 192.168.7.32/28 48-63 = Network 192.168.7.48/28 64-79 = Network 192.168.7.64/28 80-95 = Network 192.168.7.80/28 and so on .. for 192.168.7.62, 62 falls in the range 48-63. Thus, 192.168.7.62 is part of the subnet 192.168.7.48/28. Step 3: Find the Subnet Range for 192.168.7.70/28 IP Address: 192.168.7.70 Subnet Mask: 255.255.255.240 Similarly, 192.168.7.70 is part of the subnet 192.168.7.64/28. Step 4: Compare the Subnets 192.168.7.62 is in the 192.168.7.48/28 subnet. 192.168.7.70 is in the 192.168.7.64/28 subnet. These are different subnets. Question : 88.91.0.113/11 and 88.95.45.1/11 Solution : Step 1: /11 Subnet Mask A /11 subnet mask is 255.224.0.0/11 The first 11 bits represent the network portion and 21 bits represent the host portion. Step 2: Find the Subnet Range for 88.91.0.113/11 IP Address: 88.91.0.113 Subnet Mask: 255.240.0.0 0-31 = Network 88.91.0.0/11 32-63 = Network 88.91.32.0/11 64-127 = Network 88.91.64.0/11 and so on .. for 88.91.0.113, 113 falls in the range 64-127. Thus, 88.91.0.113 is part of the subnet 88.91.64.0/11 Step 3: Find the Subnet Range for 88.95.45.1/11 IP Address: 88.95.45.1 Similarly, 88.95.45.1 is part of the subnet 88.95.45.0/11 Step 4: Compare the Subnets 88.91.0.113 is in 88.91.64.0/11 subnet. 88.95.45.1 is in the 88.95.45.0/11 subnet. These are different subnets. Question 4.0.0.6/16 and 4.0.5.2/16 Solution : Step 1: /16 Subnet Mask A /16 subnet mask is 255.255.0.0/16 The first 16 bits represent the network portion and 16 bits represent the host portion. Step 2: Find the Subnet Range for 4.0.0.6/16 IP Address: 4.0.0.0 Subnet Mask: 255.255.0.0 0 = Network 4.0.0.0/16 1 = Network 4.0.1.0/16 2 = Network 4.0.2.0/16 3 = Network 4.0.3.0/16 4 = Network 4.0.4.0/16 5= Network 4.0.5.0/16 and so on .. for 4.0.0.6, 6 falls in the range 4.0.0.0/16. Step 3: Find the Subnet Range 4.0.5.2/16 4.0.5.2 is part of the subnet 4.0.0.0/16. These belongs to same subnets 4.0 as both network octat have same bits. Question 155.19.161.2/22 and 155.19.141.7/22 Step 1: Understand the /22 subnet Subnet mask: 255.255.152.0 Binary: 11111111.11111111.11111100.00000000 the first 22 bits as the network portion. Step 2: Find the network address for each host bitwise AND between the IP and the subnet mask. IP: 155.19.161.2 Binary: 155 = 10011011 19 = 00010011 161 = 10100000 Combined (first 3 octets): 100110110001001110100000 IP: 155.19.141.7 Binary: 155 = 10011011 19 = 00011011 141 = 10001100 Combined: 100110110001101110001100 Step 3: Compare the first 22 bits Now extract the first 22 bits from both IPs: 155.19.161.2: 10011011.00010011.10100000 155.19.141.7: 10011011.00010011.10001100 They differ in the 3rd octet bit. They are not in the same subnet. Question 201.251.199.21/26 and 201.251.199.55/26 Step 1: Understand the /26 subnet Subnet mask: 255.255.255.192 Binary: 11111111.11111111.11111111.11000000 the first 26 bits as the network portion. Step 2: Find the network address for each host bitwise AND between the IP and the subnet mask. IP: 201.251.199.21 Binary: 201 = 11001001 251 = 11111011 199 = 11000111 21 = 00010101 Combined : 11001001.11111011.11000111.00010101 IP: 201.251.199.55 Binary: 201 = 11001001 251 = 11111011 199 = 11000111 55 = 00110111 Combined: 11001001.11111011.11000111.00110111 Step 3: Compare the first 26 bits Now extract the first 26 bits from both IPs: 201.251.199.21: 11001001.11111011.11000111.00010101 201.251.199.55: 11001001.11111011.11000111.00110111 26 bits are same for both IP’s. Both are in same subnet. Question 181.14.22.7/25 and 181.14.22.102/25 Solution: 181.14.22.7: 10110101.00001110.00010110.00000111 181.14.22.102: 10110101.00001110.00010110.01100110 first 25 bits of both IPs are the same. Both belong to same subnet.

- Supernetting Class C Networks (2 marks) - Figure out the subnet mask required for the following network that will be supernetted. 192.168.88.0/24 192.168.89.0/24 192.168.90.0/24 192.168.91.0/24 Convert IP’s to binary: 192.168.88.0 : 11000000.10101000.01011000.00000000 192.168.89.0: 11000000.10101000.01011001.00000000 192.168.90.0: 11000000.10101000.01011010.00000000 192.168.91.0: 11000000.10101000.01011011.00000000 AND 11111111.11111111.11111100.00000000 Binary : 11000000.10101000.01011000.00000000 Bits till 22 are same will be doing AND operation with 1 Remaining 10 bits will be doing AND operation with 0 Supernet CIDR: 192.168.88.0/22 Network Address: 192.168.88.0 Subnet Mask : 255.255.252.0 - Figure out the broadcast ID for the supernetted network in a. Broadcast Address : 192.168.91.255